[image: image1.png]



ICLOROX

The Clorox Company
1221 Broadway Oakland, California 94612 Phone: 510-271-7000

Material Safety Data Sheet
	Product:                lemon fresh pine-sol® cleaner & antibacterial spray
	

	Description:            clear yellow liquid with citrus odor
	

	Other Designations                           Distributor                    Emergency Telephone Nos.
	

	Clorox Sales Company                                    For Medical Emergencies, call EPA Registration Number 581 3-55                                        1221 Broadway                            Rocky Mountain Poison Center:  1-800-446-1014 Oakland, CA 94612                                For Transportation Emergencies, call Chemtrec:   1-800-424-9300
	

	II Health Hazard Data
	Ill Hazardous Ingredients
	

	Eye irritant. Direct or prolonged skin contact may result in irritation. Avoid contact with food.
Overexposure to this product may cause the Worker Exposure Limit of 2-butoxyethanol to be exceeded.   2-ButoxyethanoT has been shown to cause red blood cell hemolysis in laboratory animals and secondary injury to the kidney and liver.   However, humans appear to be resistant to this effect.
No medical conditions are known to be aggravated by exposure to this product.
FIRST AID:
	Ingredient                           Concentration         Worker Exposure Limit
	

	
	Alkyl(C,240%;C,450%;            0.3%                 Not established. C,610%) dimethyl benzyl
	

	
	ammonium chloride CAS # 68424-85-1
2-Butoxyethanol                        1 - 5%                25 ppm - skin TLV-TWA" CAS #111 -76-2                                               50 ppm - skin PEL"
Note that 2-butoxyethanol can be absorbed through the skin.   This may contribute to overall exposure.
'TLV-TWA = ACGIH Threshold Limit Value - Time Weighted Average
	

	EYE CONTACT:    Flush eyes with plenty of water for 15 minutes.    If
	
	

	irritation occurs, call a physician. SKIN CONTACT:    Wash skin with water.    If irritation persists, call a
	
	

	physician. INGESTION:  Drink a glassful of water. Call a physician.
	None of the materials in this product are on the IARC, OSHA, or NTP carcinogen lists.
	

	IHALATION:   Remove to fresh air.   If respiratory irritation develops or
	
	

	ureathlng becomes difficult, call a physician.
	
	

	IV Special Protection and Precautions
	V Transportation and Regulatory Data
	

	Hygienic Practices: Wash hands after direct contact.
	DOT/IATA/IMO:  Not restricted.
	

	Engineering Controls:  Use local exhaust to minimize exposure to product
	TSCA Status: All components of this product are on the TSCA inventory.
	

	mist. Personal Protective Equipment:   Wear safety glasses.   Wear rubber or
	EPA - SARA Title III/CERCLA:   Packaged product \s not reportable under
	

	
	Sections 31 1/312. This product contains no chemicals which are regulated under Section 313 and contains glycol ethers (<5%) and sodium hydroxide (CAS    #1310-73-2,     <0.5%),    which    are    regulated    under
c;srtirtn ^n^'PFOP1 A
	

	neoprene gloves if there is the potential for repeated or prolonged skin contact.
	
	

	
	
	

	VI Spill Procedures/Waste Disposal
	VII Reactivity Data
	

	Spill Procedures:    Absorb and containerize.    Wash residual down to
	Stable under normal use and storage conditions.
	

	sanitary sewer. Contact the sanitary treatment facility in advance to assure ability to process washed-down material.
Waste Disposal:   Dispose of in accordance with all applicable federal.
	
	

	state, and local regulations.
	
	

	VIII  Fire and Explosion Data
	IX  Physical Data
	

	Flash Point:  >200°F (closed cup)
	pH    .                                                            
   -12.5
	

	Fire Extinguishing Agents:   Dry chemical, carbon dioxide (CO?), foam or
	Specific gravity                                          
  -1 .0
	

	
	Solubility in water                                    
 Soluble
	

	water spray.
	
	

	
	
	


C1983. 1991 THE CLOROX COMPANY DATA SUPPLIED IS FOR USE ONLY IN CONNECTION WITH OCCUPATIONAL SAFETY AND HEALTH

DATE PREPARED    1 0/97
